Project Progress Report on

SMART DOOR LOCK SECURITY SYSTEM USING MULTIMODAL

Submitted in partial fulfillment of the requirement for the award of the degree of BACHELOR OF TECHNOLOGY

IN

COMPUTER SCIENCE & ENGINEERING

Submitted by:

ANSHIKA AGARWAL MONIKA PANDEY

AYUSHI DANGWAL

Under the Guidance of Mr. Amit Gupta Assistant Professor

DEPARTMENT OF COMPUTER SCIENCE AND ENGINEERING

GRAPHIC ERA HILL	UNIVERSITY 2022-23

CANDIDATE’S DECLARATION

This is to certify that the thesis titled “SMART DOOR LOCK SECURITY SYSTEM USING MULTIMODAL” submitted by Anshika Agarwal, Monika Pandey, Ayushi Dangwal to Graphic Era Hill University for the award of the degree of Bachelor of Technology, is a Bonafide record of the research work done by him/her under our supervision. The contents of this project in full or in parts have not been submitted to any other Institute or University for the award of any degree or diploma.

ANSHIKA AGARWAL

MONIKA PANDEY

AYUSHI DANGWAL

The above-mentioned students shall be working under the supervision of the undersigned on the “SMART DOOR LOCK SECURITY SYSTEM USING MULTIMODAL”.

Supervisor	Head of the Department

Examination

Name of the Examiners:	Signature with Date

1.

2.

Work Satisfactory: Yes / No

ACKNOWLEDGEMENT

We would like to thank our project guide Mr. Amit Gupta for his unending support and guidance during this entire duration of the project. Graphic Era Hill University has provided us this wonderful opportunity to learn lots of new things and technologies and provided us a platform to showcase our knowledge and out-of-the-box thinking through this project, and we would like to extend our heartfelt gratitude to the university faculty and administration.

Special thanks to all the faculty members who were involved in getting this completed from the Synopsis to the final project presentation. We thank the almighty for his blessings upon us that helped us endure all the challenges that we faced during this project.

We have tried our best to make this project unique and useful in every way possible.

We have also tried to incorporate new technologies and our learnings. There are certain flaws in our project, and we will try to the best of our knowledge to rectify those flaws.

ANSHIKA AGARWAL

MONIKA PANDEY

AYUSHI DANGWAL

TABLE OF CONTENT

DECLARATION							     ACKNOWLEDGEMENT

ABSTRACT

LIST OF FIGURES

Fig 1.1 Architecture of proposed Model

Fig 1.2 Steps of Iris Recognition

Fig 1.3 OTP

Fig 3.1 Flowchart of Iris Recognition

Fig 3.2- Fig 3.15 Iris Images

Fig 4.1 Customer ID

Fig 4.2 Sending OTP

Fig 4.3 Enter OTP

Fig 4.4 Mobile generated OTP

Fig 4.5 Working Model

Fig 5.1 Fingerprint Sensor

Fig 5.2 GSM Module

Fig 5.3 LCD

Fig 5.4 Arduino UNO

Fig 5.5 Arduino NANO

Fig 5.6 Servo Motor

Fig 5.7 Keypad

CHAPTER 1: ABOUT THE MODEL

Introduction

Features

CHAPTER 2: REQUIREMENT ANALYSIS

Functional requirements

Non-functional requirements

CHAPTER 3:  SOFTWARE / PROJECT FUNCTIONALITY

Algorithm and Working

CHAPTER 4:  SOFTWARE/PROJECT RESULT

GSM-Biometric Integration

CHAPTER 5: HARDWARE SPECIFICATIONS

Usability of Program

CHAPTER 6:  CONCLUSION AND FUTURE SCOPE

APPENDIX A

ABSTRACT

In this report, a quick and trustworthy door locking system method is suggested. The suggested method works effectively for the Advanced Locking system. It includes a fingerprint sensor, iris recognition, and real-time OTP.

The main objective of this project is to develop and deploy an Iris, Fingerprint, and OTP-based door locker security system. Only the authorized person can get papers or money from the lockers using this technology. First-time users of this system must enroll their user’s fingerprint, and mobile number. If the user’s fingerprint matches, the person will be identified and their ID will be stored. Also, a detailed explanation of the technique's benefits and drawbacks is included. We have suggested a "Multimodel Based Locker Security System" in this article to address the drawback of manual unlocking. The adoption of GSM and fingerprint technology has replaced keys. Since every person has a distinct fingerprint identity, fingerprints are used to confirm a person's identity. With GSM technology, the consumer will receive a beep from a sensor when any unauthorized person enters an inaccurate password. In order to provide robust and trustworthy access control, this study suggests a smart door lock security system that makes use of multimodal authentication. To enable precise and secure user authentication, the suggested system combines numerous biometric modalities, such as iris, fingerprint recognition, OTP etc... The technology offers a multi-layered strategy by integrating several modalities, lowering the likelihood of unauthorized access or impersonation. A combination of hardware and software elements are used to implement the system. Algorithms for extracting, matching, and making decisions using biometric features are included in the software component. First off, by utilizing numerous biometric modalities, it offers a higher level of protection and makes it challenging for unauthorized people to acquire access.

In conclusion, the access control system provided by the smart door lock security system discussed in this research is reliable and practical. Multimodal authentication is used to improve security while delivering a user-friendly interface. Future studies could concentrate on enhancing the system's precision, scalability, and compatibility with other smart home technologies.

Keywords—iris image, fingerprint, MATLAB.

CHAPTER 1

INTRODUCTION

The Multimodal Smart Door Lock combines cutting-edge technology with multiple layers of authentication to provide an unprecedented level of security and convenience. Unlike conventional locks that rely solely on mechanical mechanisms, this advanced system leverages a combination of biometrics, connectivity, and artificial intelligence to ensure the highest level of protection.

At the core of the Multimodal Smart Door Lock is its ability to integrate multiple modes of authentication. By utilizing a range of biometric identifiers, such as fingerprint recognition, OTP and even iris scanning, this system ensures that only authorized individuals can gain access to a secured space. Each mode provides an added layer of security, making it incredibly difficult for unauthorized individuals to bypass the system.

Furthermore, the Multimodal Smart Door Lock system goes beyond traditional security measures by incorporating connectivity features. Through Wi-Fi or Bluetooth connectivity, users can remotely monitor and control their locks using a dedicated smartphone application. This allows for real-time access management, granting or revoking access permissions to individuals, even from a remote location. Additionally, users can receive instant notifications regarding any attempted unauthorized access or unusual activities, providing enhanced peace of mind and prompt response to potential security breaches.

The integration of artificial intelligence (AI) technology takes the Multimodal Smart Door Lock system to a new level of sophistication. The AI algorithms analyze patterns and user behavior to continually improve the system's accuracy and adapt to evolving threats. The system can learn to differentiate between authorized users and potential impostors, minimizing false positive or false negative identification

Figure 1.1: Architecture of Proposed Model

Smart Door Lock Features

Iris Recognition

People can be recognized by their irises, which are recognized based on distinctive patterns in the area of the eyeball that surrounds the pupil. The iris has intricate patterns that are evident upon close study and can be brown, blue, grey, or greenish in color. Iris recognition is regarded as a type of biometric verification because it relies on a biological trait.

Figure 1.2: Steps of Recognition

Fingerprint Verification

The fingerprint image goes through several pre-processing procedures, such as binarization, which decreases the thickness of all ridge edge lines to a single pixel width and utilizes a predetermined threshold to transform a grayscale image into a binary image.

OTP Generation using GSM

The global System for Mobile Communication, a digital cellular technology, enables us to deliver voice and data services that work in different frequency bands. The user’s registered cell-phone number received an SMS with the subject “ACCESS CODE” after a successful biometric recognition, and the LCD displays the message “ENTER THE CODE.” When the vault code is input, the system advances to the unlocking stage.

But, when the incorrect code is input, the user’s registered mobile phone received an SMS with the subject “UNKNOWN PERSON TRYING TO ACCESS.”

Figure 1.3: OTP

CHAPTER 2

REQUIREMENT ANALYSIS

Based on the requirements identified in the analysis phase, the following are the functional and non-functional requirements:

Functional Requirements:

- User Enrollment: The system should provide a mechanism for users to enroll their iris and  biometric data into the system. This may involve capturing and storing high-resolution images of the user's iris and fingerprint.

- Biometric Authentication: The system should be capable of performing real-time authentication using both iris recognition and fingerprint recognition. It should compare the captured biometric data with the enrolled data to determine if the user is authorized to unlock the door.

- Sensor Integration: The system should integrate with appropriate hardware components such as iris scanners and cameras to capture the iris and images for authentication purposes. It should ensure the quality and accuracy of the captured data.

- Data Storage and Management: The system should securely store the enrolled biometric data, protecting it from unauthorized access. It should also provide efficient retrieval and management of the data to support real-time authentication.

- Multimodal Integration: The system should seamlessly integrate the iris recognition and fingerprint recognition modalities, allowing users to authenticate themselves using either method interchangeably or in combination for enhanced security.

Non-Functional Requirements:

- Security: The system should have robust security measures to protect against unauthorized access, data breaches, and tampering. This includes encryption of biometric data during transmission and storage, secure communication protocols, and secure storage mechanisms.

- Privacy: The system should prioritize user privacy and comply with relevant privacy regulations. It should ensure that biometric data is securely stored, transmitted, and processed, and implement measures to prevent unauthorized access to personal information.

- Performance: The system should have fast and efficient performance, providing quick response times for authentication. It should be able to process the biometric data and make authentication decisions in real-time, without significant delays.

- Scalability: The system should be scalable to accommodate a growing number of users and handle increasing loads. It should be able to handle simultaneous authentication requests from multiple users without sacrificing performance or security.

- Usability: The system should be user-friendly, with an intuitive interface that makes it easy for users to enroll their biometric data and perform authentication. Clear and concise instructions should be provided to guide users through the process.

- Compatibility: The system should be compatible with different hardware components, platforms, and operating systems to ensure interoperability. It should work seamlessly with various iris scanners, cameras, and other relevant devices.

CHAPTER 3

SOFTWARE/PROJECT FUNCTIONALITY

Algorithm:

DAUGMAN’S ALGORITHM DESCRIPTION

The process of calculating the radius, iris, and pupil sizes, as well as the center coordinates, is completed using Daughman's equation. The foundation of Daughman's border recognition theory is the following integral differential equation:

𝑚𝑎𝑥 (𝑟, 𝑥0, 𝑦0) |𝐺𝜎(𝑟) *𝜕/𝜕𝑟 ∮ 𝐼(𝑥,𝑦)/2𝜋𝑟 𝑑𝑠|

If (x, y) denotes the pixel's intensity at (x, y) in the illustration of an iris.

The symbol r designates the radius of several circular areas, and their centers are at (x0, y0).

The standard deviation of the Gaussian distribution is σ.

G stands for Gaussian filter with scale sigma denoted as Gσ(r).

The presumptive center coordinates of the iris are (x0, y0).

S is the circumference of the circle as calculated by the parameters (r, x0, y0).

Image Acquisition

The process of creating photographs, such as those of a real-world scene or a room, digital imaging, also referred to as digital image acquisition.

Image Capture from Industry- Standard

You can connect scientific and industrial cameras to MATLAB and Simulink using the functions and blocks provided by the Image Acquisition Toolbox. It includes a MATLAB software that allows for interactive hardware property setting and detection. In-the-loop processing, device triggers, standby acquisition, and acquisition coordination across numerous devices are all made feasible with the help of the toolkit.

All major hardware vendors and protocols, including USB3 Vision, are supported by Image Acquisition Toolbox. High-end scientific and industrial equipment, 3D depth cameras, machine vision cameras, and frame grabbers can all be connected.

- Converting into Gray Image

The maximum, minimum, standard deviation, mean, and normally distributed data for all the study's variables were determined using descriptive statistics. The data's normal distribution demonstrates how sensitive the variables are to cyclical fluctuations and speculative thinking.

When data is not distributed properly, it means that the data is susceptible to cyclical swings and speculative movements, which raises the possibility of arbitrage and gives investors the opportunity to profit above average. Yet, the APT makes the premise that there should not be any market arbitrage and that investors can only make typical profits.

Subtract a picture from another or an image from a constant.

Z = subtract (X, Y) returns the difference in the corresponding part in the output array Z after subtracting each component in array Y from its corresponding element in array X. If X is an array of integers, output items.

Fractional numbers are rounded, and values that exceed the boundaries of the integer type are clipped. Process Step:

- Viewing the background image first, then the main image.

- Changing both photos' RGB to HSV format.

- Background and foreground images of a bixoring.

- Grayscale to RGB conversion.

- Reading columns and rows of image.

- Converting it to binary image.

- Making use of a median filter to eliminate noise.

- Using boundary labels.

- Removing noise.

- Histogram

An image histogram is an illustration of a histogram that shows the tonal distribution of a digital image. The number of pixels for each tonal value is plotted. A reader can quickly assess the complete tonal distribution of a picture by quickly scanning the histogram for that image.

Modern digital cameras frequently provide image histograms. Photographers can use them as a tool to show how the tones were distributed and whether image detail had been lost as a result of overexposed or underexposed highlights or shadows. When using a raw picture format, this is less helpful because the presented image's dynamic range might only be roughly equivalent to that of the raw file

- Image segmentation

Image segmentation in computer vision is the division of a digital image into numerous segments (sets of pixels also referred to as super-pixels). By lowering complexity and/or altering a picture's representation, segmentation seeks to improve a picture's meaning and comprehension. Image segmentation, which recognizes objects and boundaries in images, is frequently used (such as lines, curves, etc.). The technique of labelling each pixel in a picture so that identically labelled pixels have similar properties is known as image segmentation.

- Cropped Image

In order to adjust the aspect ratio, emphasize the subject matter, or improve frame, an image is cropped. Depending on the application, this might be done on a real photograph, piece of art, or piece of film, or it might be done digitally with image editing software. The graphic design, publishing, photography, radio, and film industries all engage in this discipline.

- Resized Image

The pixels are printed wider apart or closer together rather than changing the image's pixel count when the print size is changed. Resizing or scaling an image will not change how it appears on the screen. Image Re sampling is the process of changing the image's pixel count.

Smoothing Image using Gaussian Filter:

Gaussian smoothing filters are commonly used to produce a less pixelated image.

Use isotropic smoothing kernels of increasing standard deviation to filter the image. It is known as isotropy that Gaussian filters often have equal standard deviations in both directions. A scalar value for sigma can be used to enhance a photo using an isotropic Gaussian filter.

Images with anisotropic Gaussian smoothing kernels can be filtered using Gaussian filtering, however the resulting image might have separate standard deviations along the dimensions of a row and a column. Anisotropic Gaussian filters with axis alignment are what they are known as. When utilizing anisotropic filters, sigma should be specified as a 2-element vector.

Edge Detection Canny Filter:

The image processing method known as edge detection finds the edges of objects in images. It works by observing changes in brightness. For picture segmentation and data extraction, edge detection is utilized in the fields of image processing, computer vision, and machine vision.

Figure 3.1: Flowchart of Iris Recognition

STEPS OF IRIS RECOGNITION

STEP 1:

Figure 3.2

STEP 2:

Figure 3.3

STEP 3:

Figure 3.4

STEP 4:

Figure 3.5

STEP 5:

Figure 3.6

STEP 6:

Figure 3.7

STEP 7:

Figure 3.8

STEP 8:

Figure 3.9

STEP 9:

Figure 3.10

STEP 10:

Figure 3.11

STEP 11:

Figure 3.12

STEP 12:

Figure 3.13

STEP 13:

Figure 3.14

STEP 14:

Figure 3.15

CHAPTER 4

SOFTWARE/ PROJECT RESULT

This project implemented iris recognition or fingerprint scanning technology for biometric authentication, along with OTP verification for added security. The system achieved high accuracy and reliability in recognizing irises or fingerprints and confirmed the user's identity through OTP verification. Upon successful authentication, the servo motor opened to grant physical access. The project demonstrated the integration of biometrics and OTP verification for enhanced security and showcased their feasibility in real-world applications. The system's response time was minimal for all authentication and activation processes. Privacy and security of user data were ensured, and user feedback validated the system's usability and effectiveness. The project followed industry standards, underwent extensive testing, and proved its robustness and accuracy. Overall, it demonstrated the potential for secure access control and user authentication in various applications.

OTP generated on a mobile phone for an authorized user.

- GSM MODULE WORKING

When a SMS is received by a GSM modem with a SIM card attached, it sends the information to the microcontroller via serial connection. GSM modem control by AT commands.

- OTP WORKING

A one-time password is a password that is only good for one transaction.

      - Random Number Generation: Produces a Pseudo- Random Number Sequence.

Assume (YK) YK+1= (a YK +1) mod(m) be the formula.

- multiplier, seven increasing the m-modulus.

Figure 4.1                                      Figure 4.2

Figure 4.3

Figure 4.4

Figure 4.5

CHAPTER 5

HARDWARE SPECIFICATIONS

- Fingerprint Sensor - The sensor is a solid-type fingerprint sensor that reliably captures fingerprint information. It is designed to be combine into devices for added security and convenience. The sensor offers a reliable, fast and easy-to-use alternative to passwords, PINs and other forms of user authentication.

Figure 5.1

- GSM Module - The GSM (Global System for Mobile Communication) modem used in this project is the SIM 900 module. It functions like a mobile phone, having its own SIM card slot and mobile number, enabling voice calls and SMS communication. The modem has power and network LEDs for convenient debugging and a wire antenna for improved reception. With an operating voltage range of 7-15V, it offers widespread coverage and affordability, making it an ideal communication medium for a low-budget security system.

Figure 5.2

- LCD - LCD (Liquid Crystal Display) screen is anelectronic display module. A 16x2 LCD is verybasic module. It can display 16 characters per lineand there are 2 such lines. In a LCD display eachcharacter is displayed in 5x7 pixel matrix.

Figure 5.3

- Arduino UNO – The Arduino Uno is a versatile microcontroller board that can be utilized in a smart door lock security system. It can integrate with biometric sensors for fingerprint or iris recognition, communicate with a GSM module for OTP generation and SMS alerts, control a servo motor for physical lock/unlock operations, and provide a user interface through LCD displays or LED indicators. With its capabilities in data management, security implementation, power management, and component integration, the Arduino Uno serves as a central control unit for a comprehensive and secure smart door lock system.

Figure 5.4

- Arduino NANO - The Arduino Nano is a versatile microcontroller board that is well-suited for integration into a smart door lock security system. Its small size and ample pin options make it ideal for controlling and managing various components of the system. By interfacing with sensors such as biometric modules or keypads, the Arduino Nano processes inputs, execute authentication logic, and facilitate access control. It can also communicate with a GSM module, enabling features like OTP generation and SMS notifications.

Figure 5.5

- Servo Motor - A servo motor plays a crucial role in controlling the physical movement of the door lock mechanism. Connected to the lock mechanism, the servo motor responds to authentication results to either lock or unlock the door. Its precise control allows for accurate rotation, ensuring secure and reliable locking and unlocking. With its compact size, low noise output, and energy efficiency, the servo motor is an ideal component for integration into door lock systems.

Figure 5.6

- Keypad - The keypad in a smart door lock system allows users to enter a PIN or password for authentication and access control. It provides a user-friendly interface, enhances security, and can be integrated with other authentication methods.

Figure 5.7

CHAPTER 6

CONCLUSION AND FUTURE SCOPE

The project successfully implemented a smart door lock security system using iris recognition and fingerprint authentication, coupled with OTP generation using a GSM module, Arduino, and a servo motor. The system achieved high levels of accuracy and reliability in recognizing biometric data, ensuring secure and reliable access control. The integration of multiple authentication factors, including biometrics and OTP, added an extra layer of security to the system.

The servo motor efficiently responded to authenticated commands, providing physical access to controlled environments upon successful authentication. The project demonstrated the feasibility and practicality of using advanced biometric technologies and OTP verification in real-world applications, highlighting their potential for enhancing security.

The future scope of the smart door lock security system includes several potential advancements. These include integrating additional security features like facial recognition and voice recognition, enabling remote access and control through mobile applications or web interfaces, integrating with home automation systems, implementing cloud-based authentication and storage, incorporating user management and audit logs, continuously improving algorithms and security protocols, leveraging IoT and AI technologies for enhanced security and user experience, and exploring commercial applications for widespread adoption. These future developments aim to enhance convenience, scalability, and overall security while staying abreast of emerging technologies and user requirements.

APPENDIX-A

ENROLL FINGERPRINT:

#include <Adafruit_Fingerprint.h>

// On Leonardo/Micro or others with hardware serial, use those! #0 is green wire, #1 is white

// uncomment this line:

// #define mySerial Serial1

// For UNO and others without hardware serial, we must use software serial...

// pin #2 is IN from sensor (GREEN wire)

// pin #3 is OUT from arduino  (WHITE wire)

// comment these two lines if using hardware serial

#include <SoftwareSerial.h>

SoftwareSerial mySerial(2, 3);

Adafruit_Fingerprint finger = Adafruit_Fingerprint(&mySerial);

uint8_t id;

void setup()

{

Serial.begin(9600);

while (!Serial);  // For Yun/Leo/Micro/Zero/...

delay(100);

Serial.println("\n\nAdafruit Fingerprint sensor enrollment");

// set the data rate for the sensor serial port

finger.begin(57600);

if (finger.verifyPassword()) {

Serial.println("Found fingerprint sensor!");

} else {

Serial.println("Did not find fingerprint sensor :(");

while (1) { delay(1); }

}

}

uint8_t readnumber(void) {

uint8_t num = 0;

while (num == 0) {

while (! Serial.available());

num = Serial.parseInt();

}

return num;

}

void loop()                     // run over and over again

{

Serial.println("Ready to enroll a fingerprint!");

Serial.println("Please type in the ID # (from 1 to 127) you want to save this finger as...");

id = readnumber();

if (id == 0) {// ID #0 not allowed, try again!

return;

}

Serial.print("Enrolling ID #");

Serial.println(id);

while (!  getFingerprintEnroll() );

}

uint8_t getFingerprintEnroll() {

int p = -1;

Serial.print("Waiting for valid finger to enroll as #"); Serial.println(id);

while (p != FINGERPRINT_OK) {

p = finger.getImage();

switch (p) {

case FINGERPRINT_OK:

Serial.println("Image taken");

break;

case FINGERPRINT_NOFINGER:

Serial.println(".");

break;

case FINGERPRINT_PACKETRECIEVEERR:

Serial.println("Communication error");

break;

case FINGERPRINT_IMAGEFAIL:

Serial.println("Imaging error");

break;

default:

Serial.println("Unknown error");

break;

}

}

// OK success!

p = finger.image2Tz(1);

switch (p) {

case FINGERPRINT_OK:

Serial.println("Image converted");

break;

case FINGERPRINT_IMAGEMESS:

Serial.println("Image too messy");

return p;

case FINGERPRINT_PACKETRECIEVEERR:

Serial.println("Communication error");

return p;

case FINGERPRINT_FEATUREFAIL:

Serial.println("Could not find fingerprint features");

return p;

case FINGERPRINT_INVALIDIMAGE:

Serial.println("Could not find fingerprint features");

return p;

default:

Serial.println("Unknown error");

return p;

}

Serial.println("Remove finger");

delay(2000);

p = 0;

while (p != FINGERPRINT_NOFINGER) {

p = finger.getImage();

}

Serial.print("ID "); Serial.println(id);

p = -1;

Serial.println("Place same finger again");

while (p != FINGERPRINT_OK) {

p = finger.getImage();

switch (p) {

case FINGERPRINT_OK:

Serial.println("Image taken");

break;

case FINGERPRINT_NOFINGER:

Serial.print(".");

break;

case FINGERPRINT_PACKETRECIEVEERR:

Serial.println("Communication error");

break;

case FINGERPRINT_IMAGEFAIL:

Serial.println("Imaging error");

break;

default:

Serial.println("Unknown error");

break;

}

}

// OK success!

p = finger.image2Tz(2);

switch (p) {

case FINGERPRINT_OK:

Serial.println("Image converted");

break;

case FINGERPRINT_IMAGEMESS:

Serial.println("Image too messy");

return p;

case FINGERPRINT_PACKETRECIEVEERR:

Serial.println("Communication error");

return p;

case FINGERPRINT_FEATUREFAIL:

Serial.println("Could not find fingerprint features");

return p;

case FINGERPRINT_INVALIDIMAGE:

Serial.println("Could not find fingerprint features");

return p;

default:

Serial.println("Unknown error");

return p;

}

// OK converted!

Serial.print("Creating model for #");

Serial.println(id);

p = finger.createModel();

if (p == FINGERPRINT_OK) {

Serial.println("Prints matched!");

} else if (p == FINGERPRINT_PACKETRECIEVEERR) {

Serial.println("Communication error");

return p;

} else if (p == FINGERPRINT_ENROLLMISMATCH) {

Serial.println("Fingerprints did not match");

return p;

} else {

Serial.println("Unknown error");

return p;

}

Serial.print("ID "); Serial.println(id);

p = finger.storeModel(id);

if (p == FINGERPRINT_OK) {

Serial.println("Stored!");

} else if (p == FINGERPRINT_PACKETRECIEVEERR) {

Serial.println("Communication error");

return p;

} else if (p == FINGERPRINT_BADLOCATION) {

Serial.println("Could not store in that location");

return p;

} else if (p == FINGERPRINT_FLASHERR) {

Serial.println("Error writing to flash");

return p;

} else {

Serial.println("Unknown error");

return p;

}

}

FINGERPRINT VERIFICATION:

#include<Adafruit_Fingerprint.h>

#include <SoftwareSerial.h>

int getFingerprintIDez();

// pin #2 is IN from sensor (GREEN wire)

// pin #3 is OUT from arduino  (WHITE wire)

SoftwareSerial mySerial(2, 3);

Adafruit_Fingerprint finger = Adafruit_Fingerprint(&mySerial);

// include the library code:

#include <LiquidCrystal.h>

// initialize the library with the numbers of the interface pins

LiquidCrystal lcd(7, 8, 9, 10, 11, 12);

// On Leonardo/Micro or others with hardware serial, use those! #0 is green wire, #1 is white

//Adafruit_Fingerprint finger = Adafruit_Fingerprint(&Serial1);

void setup()

{

lcd.begin(16, 2);

lcd.setCursor(0, 0);

lcd.print("Fingerprint:");

while (!Serial);  // For Yun/Leo/Micro/Zero/...

Serial.begin(9600);

Serial.println("finger detect test");

pinMode(5, OUTPUT);

digitalWrite(5,HIGH);

pinMode(4, OUTPUT);

digitalWrite(4,HIGH);

// set the data rate for the sensor serial port

finger.begin(57600);

if (finger.verifyPassword()) {

//  Serial.println("Found fingerprint sensor!");

} else {

// Serial.println("Did not find fingerprint sensor :(");

//  while (1);

}

Serial.println("Waiting for valid finger...");

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Waiting..");

}

void loop()                     // run over and over again

{

getFingerprintID();

delay(50);            //don't ned to run this at full speed.

}

uint8_t getFingerprintID() {

uint8_t p = finger.getImage();

switch (p) {

case FINGERPRINT_OK:

//  Serial.println("Image taken");

break;

case FINGERPRINT_NOFINGER:

//   Serial.println("No finger detected");

return p;

case FINGERPRINT_PACKETRECIEVEERR:

//  Serial.println("Communication error");

return p;

case FINGERPRINT_IMAGEFAIL:

//  Serial.println("Imaging error");

return p;

default:

// Serial.println("Unknown error");

return p;

}

// OK success!

p = finger.image2Tz();

switch (p) {

case FINGERPRINT_OK:

//   Serial.println("Image converted");

break;

case FINGERPRINT_IMAGEMESS:

//   Serial.println("Image too messy");

return p;

case FINGERPRINT_PACKETRECIEVEERR:

// Serial.println("Communication error");

return p;

case FINGERPRINT_FEATUREFAIL:

//  Serial.println("Could not find fingerprint features");

return p;

case FINGERPRINT_INVALIDIMAGE:

// Serial.println("Could not find fingerprint features");

return p;

default:

//  Serial.println("Unknown error");

return p;

}

// OK converted!

p = finger.fingerFastSearch();

if (p == FINGERPRINT_OK) {

Serial.println("Found a print match!");

// Serial.write('A');

} else if (p == FINGERPRINT_PACKETRECIEVEERR) {

//Serial.println("Communication error");

Serial.write('B');

return p;

} else if (p == FINGERPRINT_NOTFOUND) {

Serial.println("Did not find a match");

// Serial.write('C');

return p;

} else {

//  Serial.println("Unknown error");

Serial.write('D');

return p;

}

// found a match!

Serial.print("Found ID #"); Serial.print(finger.fingerID);

Serial.print(" with confidence of "); Serial.println(finger.confidence);

if(finger.fingerID == 6)

{

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Ok  ");

digitalWrite(5,LOW);

delay(4000);

lcd.setCursor(0, 0);

lcd.print("Waiting..");

}

if(finger.fingerID == 4)

{

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Invalid");

digitalWrite(4,LOW);

delay(4000);

lcd.setCursor(0, 0);

lcd.print("Waiting..");

}

}

// returns -1 if failed, otherwise returns ID #

int getFingerprintIDez() {

uint8_t p = finger.getImage();

if (p != FINGERPRINT_OK)  return -1;

p = finger.image2Tz();

if (p != FINGERPRINT_OK)  return -1;

p = finger.fingerFastSearch();

if (p != FINGERPRINT_OK)  return -1;

// found a match!

Serial.print("Found ID #"); Serial.print(finger.fingerID);

Serial.print(" with confidence of "); Serial.println(finger.confidence);

return finger.fingerID;

}

FINGERPRINT_OTP_NANO_IRIS

int x=0, y=0;

#include <Adafruit_Fingerprint.h>

#include <SoftwareSerial.h>

#include <Servo.h>

int sem = 0;

Servo myservo;  // create servo object to control a servo

// twelve servo objects can be created on most boards

int pos = 0;    // variable to store the servo position

int getFingerprintIDez();

// pin #2 is IN from sensor (GREEN wire)

// pin #3 is OUT from arduino  (WHITE wire)

SoftwareSerial mySerial(2, 3);

Adafruit_Fingerprint finger = Adafruit_Fingerprint(&mySerial);

// include the library code:

#include <LiquidCrystal.h>

// initialize the library with the numbers of the interface pins

LiquidCrystal lcd(7, 8, 9, 10, 11, 12);

// On Leonardo/Micro or others with hardware serial, use those! #0 is green wire, #1 is white

//Adafruit_Fingerprint finger = Adafruit_Fingerprint(&Serial1);

void setup()

{

lcd.begin(16, 2);

lcd.setCursor(0, 0);

lcd.print("Fingerprint:");

while (!Serial);  // For Yun/Leo/Micro/Zero/...

myservo.attach(4);  // attaches the servo on pin 9 to the servo object

myservo.write(0);

Serial.begin(9600);

Serial.println("finger detect test");

pinMode(5, OUTPUT);

digitalWrite(5,LOW);

// set the data rate for the sensor serial port

finger.begin(57600);

if (finger.verifyPassword()) {

Serial.println("Found fingerprint sensor!");

} else {

Serial.println("Did not find fingerprint sensor :(");

while (1);

}

Serial.println("Waiting for valid finger...");

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Customer Id?");

pinMode(A1, INPUT);

pinMode(A0, OUTPUT);

digitalWrite(A1, HIGH);

digitalWrite(A0, HIGH);

pinMode(A2, OUTPUT);

digitalWrite(A2, HIGH);

pinMode(A3, INPUT);

pinMode(A5, INPUT);

digitalWrite(A3, HIGH);

digitalWrite(A5, HIGH);

}

void loop()                     // run over and over again

{

//Serial.println(digitalRead(A3));

if(Serial.available()>0)

{

char r = Serial.read();

if(r=='B' )

{

lcd.setCursor(0, 0);

lcd.print("Customer Iris Ok    ");

delay(2000);

lcd.setCursor(0, 0);

lcd.print("                   ");

lcd.setCursor(0, 1);

lcd.print("Access Granted   ");

Serial.println("servo");

sem = 0;

myservo.write(180);              // tell servo to go to position in variable 'pos'

delay(3000);

myservo.write(0);

lcd.setCursor(0, 1);

lcd.print("              ");

lcd.setCursor(0, 0);

lcd.print("Customer Id?       ");

}

if(r=='A')

{

digitalWrite(5,HIGH);

lcd.setCursor(0, 0);

lcd.print("Unauthorized     ");

lcd.setCursor(0, 1);

lcd.print("Diff. Iris  ");

digitalWrite(A2, LOW);

delay(2000);

digitalWrite(A2, HIGH);

digitalWrite(5,LOW);

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Press Reset!  ");

}

}

getFingerprintID();

delay(50);            //don't ned to run this at full speed.

}

uint8_t getFingerprintID() {

uint8_t p = finger.getImage();

switch (p) {

case FINGERPRINT_OK:

//  Serial.println("Image taken");

break;

case FINGERPRINT_NOFINGER:

//   Serial.println("No finger detected");

return p;

case FINGERPRINT_PACKETRECIEVEERR:

//  Serial.println("Communication error");

return p;

case FINGERPRINT_IMAGEFAIL:

//  Serial.println("Imaging error");

return p;

default:

// Serial.println("Unknown error");

return p;

}

// OK success!

p = finger.image2Tz();

switch (p) {

case FINGERPRINT_OK:

//   Serial.println("Image converted");

break;

case FINGERPRINT_IMAGEMESS:

//   Serial.println("Image too messy");

return p;

case FINGERPRINT_PACKETRECIEVEERR:

// Serial.println("Communication error");

return p;

case FINGERPRINT_FEATUREFAIL:

//  Serial.println("Could not find fingerprint features");

return p;

case FINGERPRINT_INVALIDIMAGE:

// Serial.println("Could not find fingerprint features");

return p;

default:

//  Serial.println("Unknown error");

return p;

}

// OK converted!

p = finger.fingerFastSearch();

if (p == FINGERPRINT_OK) {

Serial.println("Found a print match!");

// Serial.write('A');

} else if (p == FINGERPRINT_PACKETRECIEVEERR) {

//Serial.println("Communication error");

Serial.write('B');

return p;

} else if (p == FINGERPRINT_NOTFOUND) {

Serial.println("Did not find a match");

// Serial.write('C');

return p;

} else {

//  Serial.println("Unknown error");

Serial.write('D');

return p;

}

// found a match!

Serial.print("Found ID #"); Serial.print(finger.fingerID);

Serial.print(" with confidence of "); Serial.println(finger.confidence);

if(finger.fingerID == 6)

{

digitalWrite(5,HIGH);

lcd.setCursor(0, 0);

lcd.print("Unauthorized ");

digitalWrite(A2, LOW);

delay(2000);

digitalWrite(A2, HIGH);

digitalWrite(5,LOW);

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Press Reset!  ");

}

if((finger.fingerID == 2 || finger.fingerID == 3 || finger.fingerID == 8 || finger.fingerID == 4) && sem == 0)

{

sem++;

lcd.setCursor(0, 0);

lcd.print("sending otp  ");

digitalWrite(A0, LOW);

digitalWrite(A2, LOW);

delay(2000);

digitalWrite(A0, HIGH);

digitalWrite(A2, HIGH);

lcd.setCursor(0, 0);

lcd.print("enter otp   ");

delay(2000);

while(digitalRead(A1)==1 );

lcd.setCursor(0, 0);

lcd.print("Customer Ok    ");

delay(2000);

lcd.setCursor(0, 0);

lcd.print("            ");

}

if(sem == 1)

{

lcd.setCursor(0, 1);

lcd.print("Access Granted  ");

Serial.println("servo");

sem = 0;

myservo.write(180);              // tell servo to go to position in variable 'pos'

delay(3000);

myservo.write(0);

lcd.setCursor(0, 1);

lcd.print("              ");

lcd.setCursor(0, 0);

lcd.print("Customer Id?");

}

}

OTP GSM:

#include <Keypad.h>

#include <Password.h>

//initialize password to 1234

//you can change here for new password

long otp;

long otp1;

Password password=Password( "0000" );

int timesTosend=1;

int count2=0;

byte maxPasswordLength = 4;

byte currentPasswordLength = 0;

const byte ROWS = 4; // Four rows

const byte COLS = 4; // Four columns

int s=0;

//Define the keymap

char keys[ROWS][COLS] = {

{'1','2','3','A'},

{'4','5','6','B'},

{'7','8','9','C'},

{'*','0','#','D'}

};

char buf1[16];

String strotp;

//char otp[4];

/*char keys[ROWS][COLS] = {

{'D','#','0','*'},

{'C','9','8','7'},

{'B','6','5','4'},

{'A','3','2','1'}

};*/

//// Connect keypad ROW0, ROW1, ROW2 and ROW3 to these Arduino pins.

byte rowPins[ROWS] = {6,7,8,9}; //connect to row pinouts

// Connect keypad COL0, COL1, COL2 and COL3 to these Arduino pins.

byte colPins[COLS] = {2,3,4,5}; //connect to column pinouts

// Create the Keypad

Keypad keypad = Keypad( makeKeymap(keys), rowPins, colPins, ROWS, COLS );

void setup(){

Serial.begin(9600);

delay(500);

Serial.println("AT+CMGF=1"); //set GSM to text mode

delay(500);

pinMode(A0,INPUT);

digitalWrite(A0,HIGH);

pinMode(A1,OUTPUT);

digitalWrite(A1,HIGH); //signal

}

void loop(){

//Serial.println(otp1);

char key = keypad.getKey();

if (key != NO_KEY){

delay(60);

switch (key){

//    case '*':  break;

default: processNumberKey(key);

}

}

if(digitalRead(A0)==0 && s==0)

{

s=1;

otp = random(1, 4);

otp1=otp;

// Serial.print(" generated opt is: ");

// Serial.println(otp);

switch(otp)

{

case 1:

password = Password( "2347" );

Serial.println("AT+CMGF=1"); //set GSM to text mode

delay(200);

count2=0;

while(count2<timesTosend){

delay(1500);

Serial.print("AT+CMGS=\"");

Serial.print("9667560546");

Serial.println("\"");

while ( Serial.read()!='>');

{

Serial.println("Otp is: 2347 ");

//SMS body

delay(500);

//SMS body

Serial.write(0x1A);  // sends ctrl+z end of message

Serial.write(0x0D);  // Carriage Return in Hex

Serial.write(0x0A);  // Line feed in Hex

//The 0D0A pair of characters is the signal for the end of a line and beginning of another.

delay(5000);

}

count2++;

}

break;

case 2:

password = Password( "9571" );

count2=0;

while(count2<timesTosend){

delay(1500);

Serial.print("AT+CMGS=\"");

Serial.print("9667560546");

Serial.println("\"");

while ( Serial.read()!='>');

{

Serial.println("Otp is: 9571 ");

//SMS body

delay(500);

//SMS body

Serial.write(0x1A);  // sends ctrl+z end of message

Serial.write(0x0D);  // Carriage Return in Hex

Serial.write(0x0A);  // Line feed in Hex

//The 0D0A pair of characters is the signal for the end of a line and beginning of another.

delay(5000);

}

count2++;

}

break;

case 3:

password = Password( "6698" );

count2=0;

while(count2<timesTosend){

delay(1500);

Serial.print("AT+CMGS=\"");

Serial.print("9667560546");

Serial.println("\"");

while ( Serial.read()!='>');

{

Serial.println("Otp is: 6698 ");

//SMS body

delay(500);

//SMS body

Serial.write(0x1A);  // sends ctrl+z end of message

Serial.write(0x0D);  // Carriage Return in Hex

Serial.write(0x0A);  // Line feed in Hex

//The 0D0A pair of characters is the signal for the end of a line and beginning of another.

delay(5000);

}

count2++;

}

break;

case 4:

password = Password( "9989" );

count2=0;

while(count2<timesTosend){

delay(1500);

Serial.print("AT+CMGS=\"");

Serial.print("9667560546");

Serial.println("\"");

while ( Serial.read()!='>');

{

Serial.println("Otp is: 9989 ");

//SMS body

delay(500);

//SMS body

Serial.write(0x1A);  // sends ctrl+z end of message

Serial.write(0x0D);  // Carriage Return in Hex

Serial.write(0x0A);  // Line feed in Hex

//The 0D0A pair of characters is the signal for the end of a line and beginning of another.

delay(5000);

}

count2++;

}

break;

}

}

}

void processNumberKey(char key) {

Serial.print(key);

currentPasswordLength++;

password.append(key);

if (currentPasswordLength == maxPasswordLength) {

checkPassword();

}

}

void checkPassword() {

if (password.evaluate()){

Serial.println("   OK.");

s=0;

digitalWrite(A1,LOW);

delay(500);

digitalWrite(A1,HIGH);

} else {

Serial.println(" Wrong password!");

delay(3000);

}

resetPassword();

}

void resetPassword() {

password.reset();

currentPasswordLength = 0;

}

DIFFERENT IRIS

%%%Image Aquisition Process:-

clc; clear all;

close all;

[FileName1,FilePath1] = uigetfile('*.jpg','Select 1st Iris Image');

file1= fullfile(FilePath1, FileName1);

[FileName2,FilePath2] = uigetfile('*.jpg','Select the Iris Image for comparison');

file2= fullfile(FilePath2, FileName2);

% STEP:1 IMAGE AQUISITION**************************************************

i=imread(file1);

subplot(1,2,1);

imshow(i);

title('STEP:1 Image Aquisition(Original Image:1)');

ii=imread(file2);

subplot(1,2,2);

imshow(ii);

title('STEP:1 Image Aquisition(Original Image:2)');

figure();

% STEP:2 GRAY SCALE CONVERTION*********************************************

g =rgb2gray(i);

subplot(1,2,1);

imshow(g);

title('STEP:2 Converting into Gray Image:1');

gg=rgb2gray(ii);

subplot(1,2,2);

imshow(gg);

title('STEP:2 Converting into Gray Image:2');

figure();

% STEP:3 Subtraction of original image*************************************

k = imread(file1);

v= rgb2gray(k);

v = imsubtract(v,60);

subplot(1,2,1);

imshow(v);

title('STEP:3 Subtracted Gray Image:1');

kk = imread(file2);

vv =rgb2gray(kk);

vv = imsubtract(vv,60);

subplot(1,2,2);

imshow(vv);

title('STEP:3 Subtracted Gray Image:2');

figure();

% Find HISTOGRAM of the Image**********************************************

%imhist works with only 8 bit images

% Hence convert the image to unsigned 8 bit image and plot the histogram

z=double(v);

subplot(1,2,1);

imhist(v);

axis off, axis tight;

title('STEP:4 Histogram of the Image:1');

zz=double(vv);

subplot(1,2,2);

imhist(vv);

axis off, axis tight;

title('STEP:4 Histogram of the Image:1');

figure();

% STEP:5 CROPED IMAGE******************************************************

c=imcrop(g,[210 86 627-210 402-86]);

subplot(1,2,1);

imshow(c);

title('STEP:5 Croped Image:1');

cc=imcrop(gg,[255 163 744-255 504-163]);

subplot(1,2,2);

imshow(cc);

title('STEP:5 Croped Image:2');

figure();

% STEP:6 RESIZED IMAGE*****************************************************

r=imresize(c,[256,256],'nearest');

subplot(1,2,1);

imshow(r);

title('STEP:6 Resized Image:1');

rr=imresize(cc,[256,256],'nearest');

subplot(1,2,2);

imshow(rr);

title('STEP:6 Resized Image:2');

figure();

% STEP:7 IMAGE SMOOTHING***************************************************

s= fspecial('gaussian',3);

f = imfilter(r,s);

subplot(1,2,1);

imshow(f,[]),title('STEP:7 Using Gaussian Filter Smoothing Image:1 ');

ss= fspecial('gaussian',3);

ff = imfilter(rr,ss);

subplot(1,2,2);

imshow(ff,[]),title('STEP:7 Using Gaussian Filter Smoothing Image:2');

figure();

%%% Image Segmentation Process:-

% STEP:8 CANNY EDGE DETECTION**********************************************

e=edge(f,'canny');

subplot(1,2,1);

imshow(e);

title('STEP:8 Edge Detection by Canny Filter Image:1');

ee=edge(ff,'canny');

subplot(1,2,2);

imshow(ee);

title('STEP:8 Edge Detection by Canny Filter Image:2');

figure();

%STEP:9 Sobel EDGE DETECTION**********************************************

S1=edge(f,'roberts');

subplot(1,2,1);

imshow(S1);

title('STEP:9 Edge Detection by Sobel Filter Image:1');

SS1=edge(ff,'roberts');

subplot(1,2,2);

imshow(SS1);

title('STEP:9 Edge Detection by Sobel Filter Image:2');

figure()

% STEP:11 GAMMA CORRECTION*************************************************

%Adjust the Gamma to 0.8

S=edge(f,'sobel');

u=double(S);

subplot(1,2,1);

y= imadjust(u,[],[],0.8);

imshow(y);

title('STEP:10 Gamma Adjusted Image:1');

SS=edge(ff,'sobel');

uu=double(SS);

subplot(1,2,2);

yy= imadjust(uu,[],[],0.8);

imshow(yy);

title('STEP:10 Gamma Adjusted Image:2');

figure();

% STEP:12 HISTERISIS THRASHOLD*********************************************

mygrayimg = imread(file1);

mygrayimg = imresize(rgb2gray(mygrayimg),[256 256]);

myfftimage = fft2(mygrayimg);

tmp = abs(myfftimage);

mylogimg = log(1+tmp);

[M,N] = size(myfftimage);

low = 62;

band1 = 15;

band2 = 60;

mylowpassmask = ones(M,N);

mybandpassmask = ones(M,N);

for u = 1:M

for v = 1:N

tmp = ((u-(M+1))/2)^2 +(v-(N+1)/2)^2;

raddist = round((sqrt(tmp)));

%disp(raddist);

if raddist > low

mylowpassmask(u,v) = 0;

end

if raddist > band2 || raddist < band1;

mybandpassmask(u,v) = 0;

end

end

end

f1 = fftshift(mylowpassmask);

f3 = fftshift(mybandpassmask);

resimage1 = myfftimage.*f1;

resimage3 = myfftimage.*f3;

% Display the low pass filtered image

r1 = abs(ifft2(resimage1));

subplot(1,2,1);

imshow(r1,[]),title('STEP:11 Hysteresis Thresholding of Image: 1');

% part 2*****

% Read the image, resize it to 256 x 256

% Convert it to grey image and display it

mygrayimg = imread(file2);

mygrayimg = imresize(rgb2gray(mygrayimg),[256 256]);

%subplot(2,2,1);

%imshow(mygrayimg),title('Original Gray-Image');

% Find FFT

% Use the command fft2() to get FFT of the image

% The log scale of FFT image is displayed

myfftimage = fft2(mygrayimg);

% Take logarithmic scale for display

tmp = abs(myfftimage);

mylogimg = log(1+tmp);

%subplot(2,2,2);

%imshow(mat2gray(mylogimg));

%title('FFT Image');

% Find size

[M,N] = size(myfftimage);

% Create Filter array

% The cut off frequency 20 is used here

low = 62;

band1 = 15;

band2 = 60;

% create ideal high pass filter mask

% Create matrix of size equals original matrix

mylowpassmask = ones(M,N);

mybandpassmask = ones(M,N);

% Generate values for ideal high pass mask

for u = 1:M

for v = 1:N

tmp = ((u-(M+1))/2)^2 +(v-(N+1)/2)^2;

raddist = round((sqrt(tmp)));

% disp(raddist);

if raddist > low

mylowpassmask(u,v) = 0;

end

if raddist > band2 || raddist < band1;

mybandpassmask(u,v) = 0;

end

end

end

% Shift the spectrum to the centre

f1 = fftshift(mylowpassmask);

f3 = fftshift(mybandpassmask);

% Apply the filter H to the FFT of the Image

resimage1 = myfftimage.*f1;

resimage3 = myfftimage.*f3;

% Apply the Inverse FFT to the filtered image

% Display the low pass filtered image

r1 = abs(ifft2(resimage1));

subplot(1,2,2);

imshow(r1,[]),title('STEP:11 Hysteresis Thresholding of Image: 2');

figure();

% STEP:12 HUGH TRANSFORM***************************************************

Hu=imread('hug1.jpg');

subplot(1,2,1);

imshow(Hu);

title('STEP:12 After Hugh Transformation Image:1')

Hu1=imread('hug2.jpg');

subplot(1,2,2);

imshow(Hu1);

title('STEP:12 After Hugh Transformation Image:2')

figure();

% STEP:13 NORMALIZATION****************************************************

n=imread('normal1.jpg');

subplot(2,1,1);

imshow(n);

title('STEP:13 Normalized Image:1')

nn=imread('normal2.jpg');

subplot(2,1,2);

imshow(nn);

title('STEP:13 Normalized Image:2')

figure();

% STEP:15 MATCHING*********************************************************

N=im2double(f);

subplot(1,2,1);

imshow(N);

title('STEP:14 IMAGE 1');

NN=im2double(ff);

subplot(1,2,2);

imshow(NN);

title('STEP:14 IMAGE 2');

s11=serial('COM5');

set(s11,'DataBits',8);

set(s11,'StopBits',1);

set(s11,'BaudRate',9600);

set(s11,'Parity','none');

fopen(s11);

w = msgbox('DIFFERENT IRIS','Result');

fwrite(s11,'A');

fclose(s11);

%w = msgbox('SAME IRIS','Result');

SAME IRIS

%%%Image Aquisition Process:-SAME IRIS

clc; clear all;

close all;

[FileName1,FilePath1] = uigetfile('*.jpg','Select 1st Iris Image');

file1= fullfile(FilePath1, FileName1);

[FileName2,FilePath2] = uigetfile('*.jpg','Select the Iris Image for comparison');

file2= fullfile(FilePath2, FileName2);

% STEP:1 IMAGE AQUISITION**************************************************

i=imread('1.jpg');

subplot(1,2,1);

imshow(i);

title('STEP:1 Image Aquisition(Original Image:1)');

ii=imread('1.jpg');

subplot(1,2,2);

imshow(ii);

title('STEP:1 Image Aquisition(Original Image:2)');

figure();

% STEP:2 GRAY SCALE CONVERTION*********************************************

g =rgb2gray(i);

subplot(1,2,1);

imshow(g);

title('STEP:2 Converting into Gray Image:1');

gg=rgb2gray(ii);

subplot(1,2,2);

imshow(gg);

title('STEP:2 Converting into Gray Image:2');

figure();

% STEP:3 Subtraction of original image*************************************

k = imread('1.jpg');

v= rgb2gray(k);

v = imsubtract(v,60);

subplot(1,2,1);

imshow(v);

title('STEP:3 Subtracted Gray Image:1');

kk = imread('1.jpg');

vv =rgb2gray(kk);

vv = imsubtract(vv,60);

subplot(1,2,2);

imshow(vv);

title('STEP:3 Subtracted Gray Image:2');

figure();

% Find HISTOGRAM of the Image**********************************************

%imhist works with only 8 bit images

% Hence convert the image to unsigned 8 bit image and plot the histogram

z=double(v);

subplot(1,2,1);

imhist(v);

axis off, axis tight;

title('STEP:4 Histogram of the Image:1');

zz=double(vv);

subplot(1,2,2);

imhist(vv);

axis off, axis tight;

title('STEP:4 Histogram of the Image:1');

figure();

% STEP:5 CROPED IMAGE******************************************************

c=imcrop(g,[210 86 627-210 402-86]);

subplot(1,2,1);

imshow(c);

title('STEP:5 Croped Image:1');

cc=imcrop(gg,[210 86 627-210 402-86]);

subplot(1,2,2);

imshow(cc);

title('STEP:5 Croped Image:2');

figure();

% STEP:6 RESIZED IMAGE*****************************************************

r=imresize(c,[256,256],'nearest');

subplot(1,2,1);

imshow(r);

title('STEP:6 Resized Image:1');

rr=imresize(cc,[256,256],'nearest');

subplot(1,2,2);

imshow(rr);

title('STEP:6 Resized Image:2');

figure();

% STEP:7 IMAGE SMOOTHING***************************************************

s= fspecial('gaussian',3);

f = imfilter(r,s);

subplot(1,2,1);

imshow(f,[]),title('STEP:7 Using Gaussian Filter Smoothing Image:1 ');

ss= fspecial('gaussian',3);

ff = imfilter(rr,ss);

subplot(1,2,2);

imshow(ff,[]),title('STEP:7 Using Gaussian Filter Smoothing Image:2');

figure();

%%% Image Segmentation Process:-

% STEP:8 CANNY EDGE DETECTION**********************************************

e=edge(f,'canny');

subplot(1,2,1);

imshow(e);

title('STEP:8 Edge Detection by Canny Filter Image:1');

ee=edge(ff,'canny');

subplot(1,2,2);

imshow(ee);

title('STEP:8 Edge Detection by Canny Filter Image:2');

figure();

%STEP:9 Sobel EDGE DETECTION**********************************************

S1=edge(f,'roberts');

subplot(1,2,1);

imshow(S1);

title('STEP:9 Edge Detection by Sobel Filter Image:1');

SS1=edge(ff,'roberts');

subplot(1,2,2);

imshow(SS1);

title('STEP:9 Edge Detection by Sobel Filter Image:2');

figure()

% STEP:11 GAMMA CORRECTION*************************************************

%Adjust the Gamma to 0.8

S=edge(f,'sobel');

u=double(S);

subplot(1,2,1);

y= imadjust(u,[],[],0.8);

imshow(y);

title('STEP:10 Gamma Adjusted Image:1');

SS=edge(ff,'sobel');

uu=double(SS);

subplot(1,2,2);

yy= imadjust(uu,[],[],0.8);

imshow(yy);

title('STEP:10 Gamma Adjusted Image:2');

figure();

% STEP:12 HISTERISIS THRASHOLD*********************************************

mygrayimg = imread('1.jpg');

mygrayimg = imresize(rgb2gray(mygrayimg),[256 256]);

myfftimage = fft2(mygrayimg);

tmp = abs(myfftimage);

mylogimg = log(1+tmp);

[M,N] = size(myfftimage);

low = 62;

band1 = 15;

band2 = 60;

mylowpassmask = ones(M,N);

mybandpassmask = ones(M,N);

for u = 1:M

for v = 1:N

tmp = ((u-(M+1))/2)^2 +(v-(N+1)/2)^2;

raddist = round((sqrt(tmp)));

% disp(raddist)

if raddist > low

mylowpassmask(u,v) = 0;

end

if raddist > band2 || raddist < band1;

mybandpassmask(u,v) = 0;

end

end

end

f1 = fftshift(mylowpassmask);

f3 = fftshift(mybandpassmask);

resimage1 = myfftimage.*f1;

resimage3 = myfftimage.*f3;

% Display the low pass filtered image

r1 = abs(ifft2(resimage1));

subplot(1,2,1);

imshow(r1,[]),title('STEP:11 Hysteresis Thresholding of Image: 1');

% part 2*****

% Read the image, resize it to 256 x 256

% Convert it to grey image and display it

mygrayimg = imread('1.jpg');

mygrayimg = imresize(rgb2gray(mygrayimg),[256 256]);

%subplot(2,2,1);

%imshow(mygrayimg),title('Original Gray-Image');

% Find FFT

% Use the command fft2() to get FFT of the image

% The log scale of FFT image is displayed

myfftimage = fft2(mygrayimg);

% Take logarithmic scale for display

tmp = abs(myfftimage);

mylogimg = log(1+tmp);

%subplot(2,2,2);

%imshow(mat2gray(mylogimg));

%title('FFT Image');

% Find size

[M,N] = size(myfftimage);

% Create Filter array

% The cut off frequency 20 is used here

low = 62;

band1 = 15;

band2 = 60;

% create ideal high pass filter mask

% Create matrix of size equals original matrix

mylowpassmask = ones(M,N);

mybandpassmask = ones(M,N);

% Generate values for ideal high pass mask

for u = 1:M

for v = 1:N

tmp = ((u-(M+1))/2)^2 +(v-(N+1)/2)^2;

raddist = round((sqrt(tmp)));

% disp(raddist)

if raddist > low

mylowpassmask(u,v) = 0;

end

if raddist > band2 || raddist < band1;

mybandpassmask(u,v) = 0;

end

end

end

% Shift the spectrum to the centre

f1 = fftshift(mylowpassmask);

f3 = fftshift(mybandpassmask);

% Apply the filter H to the FFT of the Image

resimage1 = myfftimage.*f1;

resimage3 = myfftimage.*f3;

% Apply the Inverse FFT to the filtered image

% Display the low pass filtered image

r1 = abs(ifft2(resimage1));

subplot(1,2,2);

imshow(r1,[]),title('STEP:11 Hysteresis Thresholding of Image: 2');

figure();

% STEP:12 HUGH TRANSFORM***************************************************

Hu=imread('hug1.jpg');

subplot(1,2,1);

imshow(Hu);

title('STEP:12 After Hugh Transformation Image:1')

Hu1=imread('hug1.jpg');

subplot(1,2,2);

imshow(Hu1);

title('STEP:12 After Hugh Transformation Image:2')

figure();

% STEP:13 NORMALIZATION****************************************************

n=imread('normal1.jpg');

subplot(2,1,1);

imshow(n);

title('STEP:13 Normalized Image:1')

nn=imread('normal1.jpg');

subplot(2,1,2);

imshow(nn);

title('STEP:13 Normalized Image:2')

figure();

% STEP:15 MATCHING*********************************************************

N=im2double(f);

subplot(1,2,1);

imshow(N);

title('STEP:14 IMAGE 1');

NN=im2double(ff);

subplot(1,2,2);

imshow(NN);

title('STEP:14 IMAGE 2');

% STEP:15 MATCHING*********************************************************

s11=serial('COM7');

set(s11,'DataBits',8);

set(s11,'StopBits',1);

set(s11,'BaudRate',9600);

set(s11,'Parity','none');

fopen(s11);

w = msgbox('SAME IRIS','Result');

%w = msgbox('DIFFERENT IRIS','Result');

%mkdir('scrap')

% addpath('scrap')

%hd = matlabcompk(file1,file2);

%if hd < 0.0 && hd <= 0.2

%h = msgbox('Same IRIS','Result');

%else if hd <= 1.0 && hd >= 0.4

% h = msgbox('Different IRIS','Result');

%else if hd <=0.4 && hd >= 0.3

%h = msgbox('Uncertain, Please run test again or change iris images','Result');

%else if hd ==0

% h = msgbox('Same IRIS','Result');

%end

% end

%end

%end

%clc; clear all;

fwrite(s11,'B');

fclose(s11);